Absolutely, Michael. Here’s your updated Bruce JS Coding Prompt for M5Cardputer Adv—expanded with all the specific findings and decisions from this session, plus a strengthened QA/verification note (brace/duplicate/unterminated checks). This is a drop-in prompt you can reuse for future iterations.

🧰 Bruce JS Coding Prompt — Cardputer Adv (Session v1.9, Updated)

Target: Single-file ES5 JavaScript app for the Bruce firmware JS interpreter on M5Cardputer Adv (ESP32-S3).

Mandatory pre-run verification (do this every time):\ Check for unbalanced braces, and for duplicate or unterminated functions (e.g., a function declared twice, or missing closing } / an incomplete statement). These were root causes of parse errors we hit in this session.

1) Environment & Syntax (ES5 only)

- Use classic ES5 only: var, function declarations/prototypes.\ Do NOT use: let/const, classes, arrow functions, template strings, for...of, Promises, async/await, setTimeout/setInterval.

- Single file; no external imports except require('<module>').

- Avoid top-level try/catch and avoid nested block comments (/* ... /* ... */ ... */). Use // line comments.

- Keep top-level structure simple and balanced to avoid parse errors.

2) Runtime & Looping

- No timers; drive all behavior via a polling main loop using now() + delay(ms).

- Keep per-loop work short; do not starve keyboard polling.

- Be memory conscious: reuse buffers/objects; avoid large closures; minimize string churn in hot paths.

- Loop cadence:

  - Recommended cap: ~22 FPS ⇒ frameMs ≈ 45 ms for steadier SPI on ST7789.

  - If you need smoother, do not exceed ~30 FPS (frameMs ≈ 33 ms).

3) Display (ST7789 240×135 via SPI)

- Prefer dirty rect updates; limit redraws to specific bands/panels.

- Full-screen clear only on init or explicit mode transitions that require it:

  - We treat shutter pulse start and pulse end as transient mode changes (see below).

- Two Phase Rendering for moving graphics every frame:

  - Erase all sprites at their previous positions (dirty rects).

  - Update physics/positions.

  - Draw all sprites at their new positions.

- Use small bitmap fonts with strong contrast; keep text bands thin (≈ 12–16 px).

- Throttled band rendering:

  - For the large countdown band (HH:MM:SS), only redraw when the displayed second changes, or when forced by mode/edit/screen transitions.

  - This dramatically reduces SPI bandwidth and flicker.

- Special UI rule (session-verified):

  - While the shutter GPIO is pulsing (HIGH), set the UI background to BLUE and do a single full-screen clear at pulse start; restore BLACK with a single full-screen clear at pulse end.

  - All other rendering remains banded/dirty-rect.

4) Input (keyboard) — robust mapping

- Poll every loop with keyboard.getKeysPressed().

- Support common key variants:

  - Digits: "0".."9", "DigitX", "KEY_X".

  - Arrows and common symbols: "ArrowLeft"|"LEFT"|"<"|",", "ArrowRight"|"RIGHT"|">"|".".

- Increment/decrement: prefer + / - (also accept = / _).

- Implement simple software repeat (timestamp based) only when truly needed.

5) Power, GPIO, Audio

- Backlight/brightness: guard calls; if display.setBacklight() or display.setBrightness() is missing, treat as no-op.

- Audio: exclude unless explicitly requested; do not assume audio.tone()/audio.beep() exists.

- GPIO actions must be non-blocking:

  - Assert pin, track elapsed time in the main loop, then release; keep UI responsive.

- GPIO safety rules (session-applied):

  - Shutter pin LOW on init and LOW on exit.

  - When cycling shutter pin (G1..G4), ensure previous pin goes LOW, then re-init the new pin as output and keep it LOW until pulsing.

  - Do not change pins or pulse width mid-pulse; apply changes after pulse completes.

6) Bruce + Cardputer Adv (confirmed behaviors)

- Battery: Bruce build exposes percent via device.getBatteryCharge(); voltage not exposed on our unit. Prefer percent; show voltage only if device.getBatteryVoltage() exists.

- Parse stability: ES5 only with balanced braces and minimal top-level complexity avoids parse failures.

- LED: internal RGB LED naming can vary across Bruce builds; defer LED control unless required, or guard aggressively.

7) UI Components & Patterns (validated)

- Banner (top band, ~20 px):

  - Left: app title.

  - Right: status (Paused / Running / Pulse) right-aligned.

    - Moving run status to the top-right solves HUD overflow.

- Countdown band (central, tall): HH:MM:SS (size 3/4), optional days line above.

- Unit row (small line under countdown): "D H M S" with underline under selected unit.

- HUD (bottom band, ≈ 16 px):

  - Shows Mode, Shots, GPIO pin (G1..G4), Pulse ms.

  - Excludes run state (now in top banner).

  - Throttle updates (e.g., only when values change or at 1 Hz).

- Controls overlay:

  - Splash on startup for 3 s (splashControls(3000)).

  - C toggles overlay any time.

  - While visible, pause the app (no animation/countdown/edits) to reduce SPI work and prevent accidental changes.

- Optional toasts (top band) for transient feedback; keep non-blocking.

8) Exit behavior

- Graceful exit on Q: stop the loop and leave the current frame on screen.

- Ensure any GPIO outputs are returned to a safe state (e.g., shutter pin LOW).

✅ Starter Scaffold (updated for this session)

// ES5 Bruce JS — Cardputer Adv scaffold (v1.9 features)

// NOTE: Verify balanced braces & no duplicate/unterminated functions before running.

var display  = require('display');

var keyboard = require('keyboard');

var gpio     = require('gpio');

var W = display.width(), H = display.height();

var BG = display.color(0,0,0), WHITE = display.color(255,255,255), ACCENT = display.color(0,180,240);

var BGPULSE = display.color(0,60,200), HUDH = 16, TITLEY = 6;

var bgIsPulse=false, showHud=true, controlsVisible=false, running=true;

var gpioPins=[1,2,3,4], SHUTTERPIN=2, PULSEMS=200;

gpio.pinMode(SHUTTERPIN,"output"); gpio.digitalWrite(SHUTTERPIN,false);

function setTextEnv(){ if (typeof display.setTextColor==="function") display.setTextColor(WHITE); if (typeof display.setTextSize==="function") display.setTextSize(1); }

function printAt(x,y,s){ if (typeof display.drawString==="function") display.drawString(s,x,y); else { if (typeof display.setCursor==="function") display.setCursor(x,y); display.print(s);} }

function clearRect(x,y,w,h,c){ display.drawFillRect(x,y,w,h,(c||BG)); }

function charW(size){ return 6; } // size-1 only here for banner/HUD

function drawTopStatus(text){

var wpx = text.length charW(1);

var x = W - (wpx + 6); if (x < 100) x = 100;

setTextEnv(); printAt(x, TITLEY, text);

}

function drawBanner(status){

clearRect(0,0,W,20,BG); setTextEnv(); printAt(6, TITLEY, "Camera Shutter Timer"); drawTopStatus(status||"Paused");

}

function drawHudLine(modeStr, shots, pin, pulseMs){

if (!showHud) return;

clearRect(0, H-HUDH, W, HUDH, BG); setTextEnv();

printAt(6, H-HUDH+2, "Mode:"+modeStr+" Shots:"+shots+" GPIO:G"+pin+" Pulse:"+pulseMs+"ms");

}

function setUiBackgroundPulse(on){

if (!!on === bgIsPulse) return; bgIsPulse = !!on; BG = on ? BGPULSE : display.color(0,0,0);

display.drawFillRect(0,0,W,H,BG); drawBanner(on?"Pulse":(/armed/false?"Running":"Paused")); // supply actual state in your app

drawHudLine("Repeat",0,SHUTTERPIN,PULSEMS); // supply actual HUD data in your app

}

// Input skeleton

function handleKeys(){

var ks = keyboard.getKeysPressed(); if (!ks||ks.length===0) return; var i;

for (i=0;i<ks.length;i++){

var k = ""+ks[i];

if (k==="Q"||k==="q"){ running=false; continue; }

if (k==="C"||k==="c"){ controlsVisible=!controlsVisible; / draw overlay / redraw bands / continue; }

if (controlsVisible) { continue; }

if (k==="H"||k==="h"){ showHud=!showHud; drawHudLine("Repeat",0,SHUTTERPIN,PULSEMS); continue; }

if (k==="G"||k==="g"){ / cycle pins G1..G4 safely / continue; }

if (k==="D"||k==="d"){ / cycle pulse durations 200/500/1000/2000/3000 */ continue; }

}

}

// Init & main loop

display.drawFillRect(0,0,W,H,BG); drawBanner("Paused"); drawHudLine("Repeat",0,SHUTTERPIN,PULSEMS);

// splashControls(3000); // optional

var frameMs=45;

while (running){

handleKeys();

// perform dirty-rect, throttled band updates, non-blocking GPIO pulse timing…

delay(frameMs);

}

gpio.digitalWrite(SHUTTERPIN,false);

``

🔩 Reusable Snippets (paste as needed)

Battery (percent fallback):

function batteryText(device){

if (device && typeof device.getBatteryVoltage==="function"){

var v = device.getBatteryVoltage();

var v2 = Math.floor(v*100+0.5)/100; return "Batt: "+v2+" V";

}

if (device && typeof device.getBatteryCharge==="function"){

var p = device.getBatteryCharge(); return "Batt: "+p+" %";

}

return "Batt: N/A";

}

Two-phase render (moving graphics):

// A: erase previous

for (i=0;i<sprites.length;i++){ eraseAtPrev(sprites[i]); }

// B: physics update

for (i=0;i<sprites.length;i++){ updatePhysics(sprites[i]); }

// C: draw new

for (i=

Non-blocking GPIO pulse (e.g., 200–3000 ms):

var pulseActive=false, pulseStart=0, PULSEMS=200;

function startPulse(pin){ pulseActive=true; pulseStart=now(); gpio.digitalWrite(pin,true); /* set BLUE BG here / }

function stepPulse(pin){

if (!pulseActive) return;

if ((now()-pulseStart)>=PULSEMS){ gpio.digitalWrite(pin  if ((now()-pulseStart)>=PULSE_MS){ gpio.digitalWrite(pin,false); pulseActive=false; / restore BLACK BG here */ }

Top-right status in banner:

function drawTopStatus(text){

var wpx = text.length * 6; // size-1 approx

var x = W - (wpx + 6); if (x < 100) x = 100;

setTextEnv(); printAt  setTextEnv(); printAt(x, TITLE_Y, text);

Repeat mode cycle timing (QoL):

// After pulse completes:

var nextWait = totalMs - PULSEMS; if (nextWait < var nextWait = totalMs - PULSEMS; if (nextWait < 1) nextWait = 1;

Cycle shutter pin safely (G1..G4):

function setPinSafe(newPin){

gpio.digitalWrite(SHUTTERPIN,false);

SHUTTERPIN=newPin; gpio.pinMode(SHUTTERPIN,"output"); gpio.digitalWrite(SHUTTERPIN,false);

}

Digit key variants:

function parseDigitKey(k){

if (k.length===1 && k>="0" && k<="9") return k.charCodeAt(0)-48;

if (k.indexOf("Digit")==0 && k.length>=6){ var ch=k[5]; if (ch>="0"&&ch<="9") return ch.charCodeAt(0)-48; }

if (k.indexOf("KEY")==0 && k.length>=5){ var ch2  if (k.indexOf("KEY")==0 && k.length>=5){ var ch2=k[4]; if (ch2>="0"&&ch2<="9") return ch2.charCodeAt(0)-48; }

return -1;

🧪 QA & Review Checklist (run for every app)

- Syntax & structure: ES5 only; single file; balanced braces; no nested block comments; no top-level try/catch.

- Functions: No duplicates and no unterminated function bodies; avoid redefining helpers like fmt2, charW, etc.

- Modules: require() only for needed modules; guard calls when module availability varies across Bruce builds.

- Looping: Uses now()/delay(); no timers; keyboard polling every loop.

- Display: Dirty rect updates; full-screen clears only on init and pulse transitions; cap @ ~22 FPS (or ≤ 30 FPS).

  - Countdown band redraw once per displayed second, or on force.

- Animation: Two-phase where applicable (erase → update → draw).

- Input: Robust key variants; +/- for inc/dec; arrows & symbols mapped; overlay C pauses logic while visible; H toggles HUD.

- UI: Banner title left, status right-aligned; HUD shows Mode, Shots, GPIO, Pulse; overlay splash 3 s.

- Power & GPIO: Non-blocking pulses; pin LOW on init & exit; safe pin cycling (LOW old pin first). Avoid changes mid-pulse.

- Repeat mode timing: Next wait = Preset − Pulse (guard >=1 ms), keeping full cycle period equal to preset.

- Battery: Prefer percent via device.getBatteryCharge(); show voltage only if getBatteryVoltage() exists.

- Exit: Q quits cleanly; leave last frame; pin returned LOW.

📌 Specific Session Findings (don’t spare the details)

- Parse errors encountered and fixed:

  - Line 170 / 382 / 440 / 470: errors traced to unbalanced braces and an unfinished line; resolved by ensuring all blocks close properly and replacing a missing statement with gpio.digitalWrite(SHUTTER_PIN, false);.

  - Lesson: Even tiny mismatches (missing } or trailing fragments) cause Bruce to report “end of input” or a misleading line number. Always run a brace/duplicate/unterminated function audit before testing.

- Display stability improvements:

  - Moving from 33 ms to 45 ms frame cap (~22 FPS) reduced tearing/flicker on ST7789.

  - Throttling the countdown band to once per displayed second minimized SPI bandwidth without harming UX.

- UI behavior changes:

  - Background BLUE during pulse (GPIO HIGH), with single full clears at pulse start/end (treated as mode changes). Rest of UI uses banded/dirty-rect rendering.

  - Run state relocated to top-right of banner (Paused/Running/Pulse) to avoid HUD overflow when showing GPIO & pulse width.

  - HUD now shows: Mode, Shots, GPIO:G#, Pulse:###ms.

- New controls & safeguards:

  - G cycles shutter pin across G1..G4 with safe re-init (LOW previous pin, configure new pin as output, keep LOW until pulsing).\ No pin changes mid-pulse.

  - D cycles pulse width across 200/500/1000/2000/3000 ms; changes applied only when not pulsing.

- Repeat mode timing QoL:

  - After pulse completes, next countdown wait set to Preset − Pulse (guarded to minimum 1 ms), so full cycle period = preset (wait + pulse). This was explicitly requested and implemented.

- Overlay & input:

  - Startup controls splash 3 s; C toggles overlay and pauses app while visible.

  - H toggles HUD; Q quits cleanly.

- Battery API reality (build note):

  - Our Bruce build exposes percent via device.getBatteryCharge(); voltage not available. Use percent and guard voltage access.

If you want this prompt packaged into a Markdown file for your repo (e.g., PROMPT.md) or a shorter “engineer’s quick checklist” version, I can generate that next.